# AS9100 Quality Management System

How Acme Aerostructures's AS9100 Quality Management System governs the CWP-700 Composite Wing Panel Production Program —
the aerospace-specific layer on top of ISO 9001 that every Tier 1 aerospace supplier must hold to
sell to an OEM like Meridian Aircraft Co., and how the program's quality artifacts trace back to it.

## What AS9100 adds on top of ISO 9001

AS9100 is built on the ISO 9001
quality-management structure but adds requirements specific to aviation, space, and defense
manufacturing: configuration management and part-number control, first article inspection,
special-process controls (welding, heat treat, non-destructive testing, composite lay-up), risk
management tied to product safety, counterfeit-parts prevention, and full lot-level traceability
from raw material to delivered part. A shop can be ISO 9001 certified without any of that; it
cannot sell structural parts to an aircraft OEM without AS9100.

## How this program's artifacts map to AS9100 clauses

| AS9100 Area | Program Artifact |
| --- | --- |
| Risk-based thinking / risk management | RAIDD Log |
| Special process & first article verification | First Article Inspection (AS9102) |
| Control of nonconforming outputs | Material Review Board |
| Supplier / external provider control | Supplier Quality Engineer role (Resource Plan) + PPAP submissions (RAIDD DEP-02) |
| Configuration & traceability | Part marking and lot traceability requirements in the Program Governance plan |
| Management review & continual improvement | Lessons Learned register + Closeout Report |

## Program-level quality roles

R. Kessler holds the AS9100 QMS Owner
role for this program (Quality Engineering Lead) for the full 18 months. J. Ferraro joins at the
start of Phase 2 as MRB Chair once production volume creates a steady stream of nonconformance
dispositions. S. Pham owns supplier quality (PPAP) for the raw-material and hardware supply chain.
See the Resource Plan for FTE% and cost
detail on each role.

This page is a program-level summary, not a certification audit record —
Acme Aerostructures's AS9100 certificate and audit history are maintained at the corporate quality
management system level, outside this individual program's artifacts.